Verification Checklist

show running-config

show running-config | begin

show running-config | section

show running-config | include

show running-config | interface

show startup-config

show mac address-table

- shows all MAC table entries of all types (dynamic, static etc.)

- Interface ID

- VLAN ID

- MAC address

- Type (static, dynamic etc.)

show mac address-table dynamic - shows all dynamically learned MAC table entries

show mac address-table dynamic vlan vlan-id - shows all dynamically learned MAC table

entries in the specified VLAN

show mac address-table dynamic address MAC-addr - shows all dynamically learned MAC

table entries with the specified MAC address

show mac address-table dynamic interface int-id - shows all dynamically learned MAC table

entries associated with that interface

show mac address-table count - shows the number of entries in the MAC table, and the

total number of remaining empty slots in the MAC table

show mac address-table aging-time - shows the global and per-VLAN aging timeout for

inactive MAC table entries

show mac address-table static - lists static MAC addresses and MAC addresses learned or

defined with port security

show mac address-table static interface int-id - lists static MAC addresses and MAC

addresses learned or defined with port-security on an interface

show mac address-table secure lists MAC addresses defined or learned on ports configured

with port security

show mac address-table secure interface int-id lists MAC addresses defined or learned on

specified port configured with port security

clear mac address-table dynamic - empties the MAC table of all dynamic entries

show interfaces

- Lists detailed status and statistical information about interfaces

- Port link/protocol status (up/up)

- Port status (connected)

- Port MAC address

- Duplex & speed (does not list if they were autonegotiated or not)

- Port counters:

- Runt: frames that did not meet the minimum frame size requirement of 64 bytes

- Giant: frames that exceeded the maximum frame size requirement of 1518 bytes

- CRC: received frames that did not pass the FCS math

- Frame: received frames that have an illegal format e.g. ending with partial byte

- Output errors: total number of packets that switch port tried to transmit, but for which

some problem occurred

- Collisions: counter of all collisions that occur when the interface is transmitting a frame

- Late collisions: subset of all collisions that happen after the 64th byte of the frame

show interfaces status

- Lists one line per interface on the switch, with basic status and operating information for

each

- Interface ID (f0/1, g0/1 etc.)

- Name (description)

- Status (notconnect, connected etc.)

- VLAN ID (1, trunk etc.)

- Duplex (full, half, auto, a-full, a-half)

- Speed (10, 100, 1000, auto, a-10, a-100, a-1000 etc.)

- Type (10/100BASE-TX, 10/100/1000BASE-TX etc.)

show interfaces vlan number - lists interface details for a specified VLAN interface

show interfaces description

- Displays one line of information per interface, with a two-item status, and includes and

description that is configured on the interfaces

- Link status

- Protocol status

- Description

show dhcp lease

- lists any information the switch acquires as a DHCP client

- Leased IP address and subnet mask

- Default-gateway address

- Lease time

- DHCP server address

show crypto key mypubkey rsa - Lists the public and shared key created for use with SSH

using the crypto key generate rsa global configuration command

show ip ssh - lists status information for the SSH server, including the SSH version

- Lists connected client MAC address and port

show ssh - lists status information for current SSH connections into and out of the local

switch

show ip default-gateway - lists the switch's setting for its IPv4 default gateway

show port-security

- lists one line per interface that summarises the port security settings for any interface on

which it is enabled

- Secure Port: name of port

- Max Secure Addresses Count: number of configured maximum addresses

- Current Addresses Count: number of current addresses

- Security Violation Count: number of security violations

- Security Action (shutdown, restrict, protect)

show port-security interface int-id

- lists an interface's port security configuration settings and security operational status

- Port Security (enabled/disabled)

- Port Status (secure-up, secure-shutdown)

- Violation Mode (protect, restrict, shutdown)

- Maximum MAC Addresses: lists number of configured maximum MAC addresses allowed

- Total MAC addresses: total number of MAC addresses currently on the interface

- Configured MAC Addresses: number of statically configured MAC addresses

- Sticky MAC Addresses: number of sticky learned MAC addresses

- Last Source Address:Vlan: last source MAC address and VLAN that the switch has received

from

- Security Violation Count: number of violations that have occurred

show port-security address

- Lists the secure MAC address table

- VLAN

- MAC address

- Type (SecureSticky)

- Ports

show interfaces switchport

- Lists information about any interface regarding administrative settings and operational

state

- Administrative Mode: configured trunking status (static access, dynamic auto, dynamic

desirable, trunk)

- Operational Mode: current operational trunking status (static access, trunk)

- Administrative Trunking Encapsulation: configured encapsulation type (ISL, 802.1Q)

- Operational Trunking Encapsulation: current encapsulation type in use (ISL, 802.1Q)

- Negotiation of Trunking: DTP status (On/Off)

- Access Mode VLAN: the VLAN in which the access port is in

- Trunking Native Mode VLAN: the untagged VLAN that the trunk port allows

- Voice VLAN: the configured voice VLANs

show interfaces trunk

- Lists information about all operation trunks (but no other interfaces), including the list of

VLANs that can be forwarded over the trunk

- Administrative mode (on, auto, desirable, off)

- Operational encapsulation type (isl, 802.1q, n-isl, n-802.1q)

- Operational trunking status (trunking)

- Native VLAN

- VLANs allowed on trunk

show vlan

- Lists detailed information about the VLAN

- VLAN name: the configured name for the VLAN

- VLAN status (active, act/unsup, act/lshut)

- Ports assigned to VLAN

- Plus additional information at the end

show vlan brief

- VLAN name: the configured name for the VLAN

- VLAN status (active, act/unsup, act/lshut)

- Ports assigned to VLAN

show vlan id vlan-id

- Lists detailed information about specified VLAN

show vlan name vlan-name

- Lists detailed information about specified VLAN

show vtp status - lists VTP configuration and status information

Router

show ip interface brief

- Lists a single line of information about each interface

- IP address (no mask)

- Method: the method by which the IP address was learnt (NVRAM, DHCP etc.)

- Link/protocol status

show ip interface

- Lists detailed IP information about each interface

- IP address and mask

- Helper-address: as set by ip helper-address command

- Outgoing access list

- Inbound access list

show interface

- Lists a large set of informational messages about each interface, or specified interface

- Link/protocol status

- MAC address

- Description

- IP address and subnet mask

- Bandwidth (as BW)

- Counters (CRC, frame, collisions etc.)

show interfaces loopback number

show protocols

- Lists information about the listed interface

- Link/protocol status

- IP address and mask

show controllers

- Lists many lines of information per interface, or for one interface, for the hardware

controller of the interface

- On serial interfaces, identifies the cable as either a DCE or DTE cable

show ip route

- Lists the router's entire routing table

- Gateway of last resort: default route

- Routing protocol code (C, D, L, S, O, R)

- Network and mask

- Administrative distance (0, 1, 90, 110, 120, 254, 255)

- Metric (hop count, cost)

- Next-hop router address

- Update timer: time since route was first learned

- Outgoing interface

show ip route connected

- Lists all connected routes from the IP routing table

show ip route static

- Lists all static routes from the IP routing table

show ip route rip

- Lists all RIP-learned routes from the IP routing table

show ip route ip-addr

- Lists detailed information about the route that a router matches for the listed IP address

- Known via

- Distance

- Metric

- Outgoing interface

show ip protocols

- Lists information about the RIP (routing protocol) configuration, plus the IP addresses of

neighbouring RIP routers from which the local router has learned routes

- Routing protocol (RIP etc.)

- Timers (update every 30s, next due in 10s, invalid after 180s, hold down 180s, flushed

after 240s)

- Version (version 2)

- Interfaces

- Autosummarisation (auto-summary)

- Maximum path (maximum-path)

- Routing for Networks (network)

- Passive interfaces (passive-interface)

- Distance (distance)

- Update timer: time since last update

show ip rip database

- Lists one line per router interface, including the IP address and interface status,

prefix/length of all best routes known to RIP on this router and connected routes for

interfaces on which RIP has been enabled

- Best RIP-learned routes

- Connected routes for RIP-enabled interfaces

- Other information about networks (e.g. auto-summary status)

show ip dhcp binding

- Lists the currently leased IP addresses on a DHCP server, along with the client identifier

and lease time information

- Leased IP address

- Client ID/MAC address

- Lease expiration

- Type (dynamic, automatic, static)

show ip dhcp pool name

- Lists the configured range of addresses in the pool, along with usage statistics and

utilisation high/low-water marks

- Total leasable addresses

- Leased addresses

- Excluded addresses

- IP address range

show ip dhcp server statistics

- Lists statistics about the requests served by the DHCP server

- Number of address pools

- Number of DHCP messages received (DHCPDISCOVER, DHCPREQUEST etc.)

- Number of DHCP messages sent (DHCPOFFER, DHCPACK etc.)

show ip dhcp conflict

- Lists IP addresses that the DHCP server found were already in use when the server tried

to lease the address to a host

- IP address

- Detection method (gratuitous ARP etc.)

- Detection time

clear ip dhcp conflict

- Removes all entries from the DHCP server's conflict list

show access-lists [ACL-no. | ACL-name]

- Shows details of configured access lists for all protocols

- ACL number/name

- Commands in ACL with sequence numbers

show ip access-lists [ACL-no. | ACL-name]

- Shows IP access lists

- Same information as show access-lists

show ip nat statistics

- Lists counters for packets and NAT table entries, as well as basic configuration

information

- Total translations

- Outside/inside interfaces

- Hits/misses

show ip nat translations

- Displays the NAT table

- Protocol

- Inside/outside global/local address/port

clear ip nat translation {* | [inside global-ip local-ip]}

- Clears all or some of the dynamic entries in the NAT table, depending on which

parameters are used

debug ip nat

- Issues a log message describing each packet whose IP address is translated with NAT

- Source/destination address translations

show ipv6 interface

- Lists IPv6 settings on an interface, including link-local and other unicast IP addresses

- Link/protocol status

- Link-local IPv6 address

- Global unicast IPv6 address

- Resident subnet

- Joined group addresses: multicast addresses that the interface is listening on

- NDP settings

show ipv6 interface brief

- Lists interface status and IPv6 addresses for each interface

- Link/protocol status

- Global unicast IPv6 address

- Link-local IPv6 address

show ipv6 route

- Lists all IPv6 routes

- Similar information to show ip route

show ipv6 route connected

- Lists just the connected IPv6 routes

show ipv6 route local

- Lists just the local IPv6 routes

show ipv6 route static

- Lists just the static IPv6 routes

show ipv6 route address

- Displays detailed information about the IPv6 route this router uses to forward packets to

the IPv6 address listed in the command

- Similar information to show ip route address

show ipv6 neighbors

- Lists the router's IPv6 neighbour table

- IPv6 address

- Age

- Link-layer address

- State (reach, stale etc.)

- Interface (outgoing)

show ipv6 routers

- Lists any neighbouring routers that advertised themselves through an NDP RA message

- Neighbouring router's link-local address

- Outgoing interface

show logging

- Lists the current logging configuration, and lists buffered log messages at the end

- Console logging level, messages logged etc.

- Monitor logging level, messages logged etc.

- Buffer logging level, messages logged etc.

- Status of count/timestamps

- Buffered messages

show clock

- Lists time-of-day and the date per the local device

- HH:MM:SS.SSS

- Timezone name

- Day of week

- Date Month Year

show ntp associations

- Shows all NTP clients and servers with which the local device is attempting to synchronise

with NTP

- Reference IP address

- Stratum of remote peer

show ntp status

- Shows current NTP client status in detail

- Synchronised/unsynchronised

- Stratum of local router (-1 stratum of remote peer)

- Reference IP address

show {cdp | lldp} neighbors

- Lists one summary line of information about each neighbour

- Device ID (hostname)

- Local interface

- Holdtime (decreases from 180 for CDP, 120 for LLDP): time until information is discarded

- Capability (R, S (B for LLDP), I etc.)

- Platform (abbreviated name)

- Port ID (neighbouring interface)

show {cdp | lldp} neighbors detail

- Lists one large set of information for every neighbour

CDP

- Device ID (hostname)

- IP address of neighbouring device

- Platform (full name)

- Capabilities (full name e.g. Router)

- Local interface and port ID (outgoing)

- IOS version

- Holdtime

- Duplex

- Advertisement version (e.g. CDPv2)

LLDP

- Port ID

- System name (Switch etc.)

- IOS version

- Time remaining

- IP address

- Autonegotiation status

- Physical media capabilities (100BaseT (FD) or (HD) etc.)

- VLAN ID

show {cdp | lldp} entry name

- Displays the same information as show {cdp | lldp} neighbors detail but only for the

named neighbour

show {cdp | lldp}

- States whether CDP or LLDP is enabled globally, and lists the default update and holdtime

timers

- Update timer (CDP default: 60 seconds, LLDP default: 30 seconds)

- Holdtime timer (CDP default: 180 seconds, LLDP default: 120 seconds)

- Status of CDPv2

show {cdp | lldp} interface

- States whether CDP or LLDP is enabled on each interface, or a single specified interface

- Link/protocol status

- Interface update timer

- Interface holdtime timer

show {cdp | lldp} traffic

- Displays global statistics for the. number of CDP or LLDP advertisements sent and

received

- Total packets output/input (total CDP advertisements output/input)

- CDPv1 advertisements output/input

- CDPv2 advertisements output/input

show flash

- Lists the names and size of the files in flash memory, as well as noting the amount of flash

memory consumed and available

- File number (highest = newest)

- Length

- Filename

- Bytes used, bytes available, total bytes

dir filesystem:[directory]

- Lists the files in the referenced file system, or file system directory

- File number

- Length

- Date

- Filename

- Total bytes, bytes free

more filesystem:name

verify /md5 filesystem:name [MD5-hash]

show license

show license feature

show license udi

show version

show history - lists the commands in the current history buffer

show vlans - lists VLAN configuration and statistics for VLAN trunks configured on routers

show ip arp - lists the router's IPv4 ARP table

clear ip arp [int-id] - removes all dynamically learned ARP table entries, or if the command

lists an IP address, removes the entry for that IP address only

no debug all - stops all debug services

ping

- Tests basic IP connectivity between two hosts

traceroute

ipconfig /all

- Lists IP settings for the interface (NIC)

- MAC address

- Link-local IPv6 address

- IP address

- Subnet mask

- Default gateway

- DNS servers

- DHCP servers

netstat -rn

- Lists the host's routing table, often listing the default router with a route to 0.0.0.0

arp -a

- Lists the host's ARP table

- IP address

- MAC address

- Type (dynamic etc.)

netsh interface ipv6 show neighbors - lists a host's IPv6 neighbour table

ndp -an - lists a host's IPv6 neighbour table

ip -6 neighbor show - lists a host's IPv6 neighbour table